Emmanuel Rodriguez Lopez

CPE301 – SPRING 2016

# Design Assignment 2

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| 1. | INITIAL CODE OF TASK 1/A |  |  |
| 2. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 2/B |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 3/C |  |  |
| 4. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 4/D |  |  |
| 5. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 5/E |  |  |
| 6. | SCHEMATICS |  |  |
| 7. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| 8. | SCREENSHOT OF EACH DEMO |  |  |
| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| 10. | GOOGLECODE LINK OF THE DA |  |  |

| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| --- | --- | --- | --- |

| 1. | INITIAL CODE OF TASK 1/A |  |  |
| --- | --- | --- | --- |

ASSEMBLY CODE FOR TASK 1

SBI	DDRC, 0

BEGIN:

LDI	R20, 0x07

STS	OCR1AH, R20

LDI	R20, 0xA1

STS	OCR1AL, R20		;ocr1a set to ¼ second if prescaler is 1024

LDI	R20, 0x00

STS	TCCR1A, R20

LDI	R20, 0x0D

STS	TCCR1B, R20		;prescaler of 1024, CTC mode

LDI	R20, 1

out	PORTC, R20

L1:

IN	R20, TIFR1

SBRS	R20, OCF1A		;polling for flag

RJMP	L1

LDI	R20, 0

STS	TCCR1B, R20		;turning off timer 1

LDI	R20, 1<<OCF1A		;clear flag

OUT	TIFR1, R20

LDI	R20, 0

OUT	PORTC, R20		;toggle pin

LDI	R20, 0x07

STS	OCR1AH, R20		;set timer again, same as before since 50% DC

LDI	R20, 0xA1

STS	OCR1AL, R20

LDI	R20, 0x00

STS	TCCR1A, R20

LDI	R20, 0x0D

STS	TCCR1B, R20

L2:

IN	R20, TIFR1		;polling for flag

SBRS	R20, OCF1A

RJMP	L2

LDI	R20, 0

STS	TCCR1B, R20		;clear flag

LDI	R20, 1<<OCF1A

OUT	TIFR1, R20

JMP	BEGIN

| 1. | INITIAL CODE OF TASK 1/B |  |  |
| --- | --- | --- | --- |

C Code for Task 1

/*

* GccApplication1.c

*

* Created: 3/4/2016 1:33:49 PM

*  Author: RODRIE2

*/

#define F_CPU 8000000UL		//setting frequency

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRC |= (1<<PORTC0);		//makes PC0 an output

PORTC= 1;			//PC0 high to start

while(1)

{

_delay_us(250000);	//1/4 sec delay

if (PORTC == 1)	//to toggle PC0

{

PORTC = 0;	//happens every 1/5 second, the period

}

else

{

PORTC = 1;

}

}

return 0;

}

| 2. | INITIAL CODE OF TASK 2/A |  |  |
| --- | --- | --- | --- |

ASSEMBLY CODE FOR TASK 2

/*

* DA2TASK3_ASM.asm

*

*  Created: 3/7/2016 5:40:38 PM

*   Author: erl25_000

*/

SBI	DDRB, 1		;all PORTB pins as outputs

SBI	DDRB, 0

SBI	DDRB, 2

SBI	DDRB, 3

SBI	DDRB, 4

SBI	DDRB, 5

SBI	DDRB, 6

SBI	DDRB, 7

LDI	R21, 0			;used for keeping track of inc rising edges

BEGIN:

LDI	R20, 0x07

STS	OCR1AH, R20

LDI	R20, 0xA1

STS	OCR1AL, R20		;set to ¼ sec

LDI	R20, 0x00

STS	TCCR1A, R20

LDI	R20, 0x0D

STS	TCCR1B, R20		;CTC mode and prescaler = 1024

INC	R21

Out	PORTB, R21		;output # of rising edges

L1:

IN	R20, TIFR1		;poll flag

SBRS	R20, OCF1A

RJMP	L1

LDI	R20, 0

STS	TCCR1B, R20

LDI	R20, 1<<OCF1A		;clear flag

OUT	TIFR1, R20

LDI	R20, 0x07

STS	OCR1AH, R20

LDI	R20, 0xA1

STS	OCR1AL, R20

LDI	R20, 0x00

STS	TCCR1A, R20

LDI	R20, 0X0D

STS	TCCR1B, R20

L2:

IN	R20, TIFR1

SBRS	R20, OCF1A

RJMP	L2

LDI	R20, 0

STS	TCCR1B, R20

LDI	R20, 1<<OCF1A

OUT	TIFR1, R20

JMP	BEGIN

| 2. | INITIAL CODE OF TASK 2/B |  |  |
| --- | --- | --- | --- |

C CODE FOR TASK 2

/*

* GccApplication1.c

*

* Created: 3/4/2016 1:33:49 PM

*  Author: RODRIE2

*/

#define F_CPU 8000000UL	//setting frequency

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRB = 0xFF;		//all of PORTB is output

PORTB = 0;

int i = 1;		//indicator for duty cycle

while(1)

{

_delay_us(250000);	//1/4 sec delay, 50% DC

if (i == 1)

{

PORTB++;	//inc every rising edge occurring every ½ sec

i = 0;

}

else

{

i = 1;

}

}

return 0;

}

| 3. | INITIAL CODE OF TASK 3/A |  |  |
| --- | --- | --- | --- |

ASSEMBLY CODE FOR TASK 3

/*

* DA2TASK3_ASM.asm

*

*  Created: 3/7/2016 5:40:38 PM

*   Author: erl25_000

*/

SBI	DDRC, 5

SBI	DDRC, 6

SBI	DDRB, 1

SBI	DDRB,0

SBI	DDRB, 2

SBI	DDRB, 3

SBI	DDRB, 4

SBI	DDRB, 5

SBI	DDRB, 6

SBI	DDRB, 7

LDI	R21, 0

LDI	R22, 0			;5th rising edge indicator

LDI	R23, 0			;10th rising edge indicator

BEGIN:

LDI	R20, 0x07

STS	OCR1AH, R20

LDI	R20, 0xA1

STS	OCR1AL, R20		; 1/4 sec of ½ sec 50% DC

LDI	R20, 0x00

STS	TCCR1A, R20

LDI	R20, 0x0D

STS	TCCR1B, R20		;CTC mode and 1024 prescaler

INC	R22

INC	R23

CPI	R22, 5			;checking for 5th rising

BRNE	CHK10

LDI	R20, 0x20

OUT	PORTC, R20		;toggling PC5

LDI	R22, 0			;resetting 5th indicator

CHK10:

CPI	R23, 10		;checking for 10th rising edge

BRNE	NORM

LDI	R20, 0x40

OUT	PORTC, R20		;toggling PC6

LDI	R23, 0			;resetting 10th indicator

NORM:

INC	R21

Out	PORTB, R21		;inc portb every rising edge

L1:

IN	R20, TIFR1		;polling  for flag

SBRS	R20, OCF1A

RJMP	L1

LDI	R20, 0

STS	TCCR1B, R20

LDI	R20, 1<<OCF1A		;clearing flag

OUT	TIFR1, R20

LDI	R20, 0x07

STS	OCR1AH, R20

LDI	R20, 0xA1

STS	OCR1AL, R20

LDI	R20, 0x00

STS	TCCR1A, R20

LDI	R20, 0X0D

STS	TCCR1B, R20

L2:

IN	R20, TIFR1

SBRS	R20, OCF1A

RJMP	L2

LDI	R20, 0

STS	TCCR1B, R20

LDI	R20, 1<<OCF1A

OUT	TIFR1, R20

JMP	BEGIN

| 3. | INITIAL CODE OF TASK 3/B |  |  |
| --- | --- | --- | --- |

C CODE FOR TASK 3

/*

* DA2TASK3_C.c

*

* Created: 3/13/2016 5:55:59 PM

*  Author: Rodriguez

*/

#define F_CPU 8000000UL	//setting frequency

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRB = 0xFF;		//all PORTB as output

DDRC = 0x60;		//PC5, PC6 as output

PORTB = 0;

//PORTC = 0;

int i = 1;		//Duty cycle indicator

int five = 0;		//5th rising edge indicator

int ten = 0;		//10th rising edge indicator

while(1)

{

_delay_us(250000);	//1/4 sec delay, 50DC of ½ sec period

if (i == 1)		//at rising edge

{

PORTB++;	//rising edge # out

five++;

ten++;

i = 0;

if (ten == 10)	//at 10th rising edge, toggle PC6, clear indicators

{

PORTC = 0x40;

ten = 0;

five = 0;

}

if (five==5)		//at 5th rising edge, toggle PC5, clear indicator

{

PORTC = 0x20;

five = 0;

}

}

else

{

i = 1;

}

}

return 0;

}

| 4. | INITIAL CODE OF TASK 4/A |  |  |
| --- | --- | --- | --- |

ASSEMBLY CODE FOR TASK 4

/*

* AssemblerApplication1.asm

*

*  Created: 3/10/2016 10:04:08 AM

*   Author: RODRIE2

*/

.org 0x00

JMP MAIN

.org 0x1C

JMP COMP_INT

MAIN:

SBI	DDRC, 5

SBI	DDRC, 6

SBI	DDRB, 1

SBI	DDRB,0

SBI	DDRB, 2

SBI	DDRB, 3

SBI	DDRB, 4

SBI	DDRB, 5

SBI	DDRB, 6

SBI	DDRB, 7

LDI	R21, 0

LDI	R18, 0

LDI	R19, 0

BEGIN:

LDI	R20, HIGH(RAMEND)

OUT	SPH, R20

LDI	R20, LOW(RAMEND)

OUT	SPH, R20

LDI	R20, 0x07		;1/2 SECOND

STS	OCR1AH, R20		;OCR1A = 07A1

LDI	R20, 0xA1

STS	OCR1AL, R20

LDI	R20, 0x00

STS	TCCR1A, R20

LDI	R20, 0x0d

STS	TCCR1B, R20

LDI	R20, (1<<OCIE1A)

sts	TIMSK1, R20

SEI

LOOP:

JMP	LOOP

COMP_INT:

INC R18

INC R19

CPI	R18, 5		;for every 5th rising edge, toggle PC5

BRNE	CHK10

LDI	R20, 0x20

OUT	PORTC, R20

LDI	R18, 0

CHK10:

CPI	R19, 10	;for every 10th rising edge, toggle PC6

BRNE	NORM

LDI	R20, 0x40

OUT	PORTC, R20

LDI	R19, 0

NORM:

INC	R21

Out	PORTB, R21	;inc PORTB every rising edge

;RESETTING CLOCK

LDI	R20, 0x0F

STS	OCR1AH, R20

LDI	R20, 0x45

STS	OCR1AL, R20

LDI	R20, 0x00

STS	TCCR1A, R20

LDI	R20, 0x0D

STS	TCCR1B, R20

RETI

| 4. | INITIAL CODE OF TASK 4/B |  |  |
| --- | --- | --- | --- |

C CODE FOR TASK 4

/*

* DA2TASK4_C.c

*

* Created: 3/13/2016 7:20:38 PM

*  Author: Rodriguez

*/

#define F_CPU 8000000UL

#include <util/delay.h>

#include <avr/io.h>

#include <avr/interrupt.h>

int i = 1, five, ten;		//indicators for DC, 5th and 10th rising edge

int main(void)

{

DDRB = 0xFF;

DDRC = 0x60;

PORTC = 0;

PORTB = 0;

TCCR1A = 0x00;

TCCR1B = 0x0D;			//CTC mode with 1028 prescaler

OCR1A = 0x07A1;			//1/4 sec

TIMSK1 |= (1<<OCIE1A);		//enable interrupt mask

sei();				/enable global interrupts

while(1)

{

//infinite loop

}

}

ISR(TIMER1_COMPA_vect)

{

if (i==1)

{

PORTB++;

five++;

ten++;

i = 0;

if (ten == 10)

{

PORTC = 0x40;

ten = 0;

five = 0;

}

if (five==5)

{

PORTC = 0x20;

five = 0;

}

}

else if (i == 0)

{

i=1;

}

// RESETTING CLOCK

TCCR1A = 0x00;

TCCR1B = 0x0D;

OCR1A = 0x07A1;

}

| 6. | SCHEMATICS |  |  |
| --- | --- | --- | --- |

| 7. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| --- | --- | --- | --- |

TASK 1:

50% duty cycle on PC0, period = 0.5 second 
The first image displays the rising edge of the waveform at ¼ second. PC0 is high. The second image shows the second half of the duty cycle where PC0 is low.

TASK 2:
Images show PORTB outputting number of rising edges. With 50% DC of ½ sec period, each once happens every 500000us.

TASK 3:
In addition to Task 2’s duties, at every 5th and 10th rising edge, PC5 and PC6 will be toggled.

TASK 4
Task 3, using interrupts.

| 8. | SCREENSHOT OF EACH DEMO |  |  |
| --- | --- | --- | --- |

The red wired circuit is the one used for all experiments.

TASK 2: Counting number of rising edges, in 8 bit form on LED bar.

………..

TASK 3 / 4: Toggle 9th led every fifth and 10th led every tenth rising edge

| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| --- | --- | --- | --- |
| https://www.youtube.com/watch?v=DXYXBecOVYg |  |  |  |
| 10. | GOOGLECODE LINK OF THE DA |  |  |
| https://github.com/rodrie2/UNLVCPE301 |  |  |  |

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Emmanuel Rodriguez Lopez