Emmanuel Rodriguez Lopez

CPE301 – SPRING 2016

# Design Assignment 5

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| 1. | INITIAL CODE OF TASK 1/A |  |  |
| 2. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 2/B |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 3/C |  |  |
| 4. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 4/D |  |  |
| 5. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 5/E |  |  |
| 6. | SCHEMATICS |  |  |
| 7. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| 8. | SCREENSHOT OF EACH DEMO |  |  |
| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| 10. | GOOGLECODE LINK OF THE DA |  |  |

| 0. | COMPONENTS LIST AND CONNECTION DIAGRAM w/ PINS |  |  |
| --- | --- | --- | --- |

TASK 1 DC MOTOR

TASK 2 STEPPER MOTOR

TASK 3 SERVO

| 1 | INITIAL CODE OF TASK 1 |  |  |
| --- | --- | --- | --- |

#define F_CPU 8000000UL

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRC = 0xFF;

while (1)

{

PORTC = 0x20;	//pc5

_delay_ms(100);

PORTC = 0x00;

_delay_ms(100);

}

return 0;

}

| 2 | INITIAL CODE OF TASK 2 |  |  |
| --- | --- | --- | --- |

#define F_CPU 8000000UL

#include <avr/io.h>

#include <util/delay.h>

int main()

{

DDRD = 0x00;

DDRC = 0xFF;

while (1)

{

while (PIND == 0x02)	//PD1 to pot, if on do following

{

PORTC = 0x6;		//fast speed

_delay_ms(10);

PORTC = 0xC;

_delay_ms(10);

PORTC = 0x9;

_delay_ms(10);

PORTC = 0x3;

_delay_ms(10);

}

//if PD1 to pot registers 0

//slower speed

PORTC = 0x6;

_delay_ms(100);

PORTC = 0xC;

_delay_ms(100);

PORTC = 0x9;

_delay_ms(100);

PORTC = 0x3;

_delay_ms(100);

}

return 0;

}

| 3 | INITIAL CODE OF TASK 3 |  |  |
| --- | --- | --- | --- |

#define F_CPU 8000000UL

#include <avr/io.h>

#include <util/delay.h>

int main()

{

DDRD = 0x00;

DDRB = 0xFF;

ICR1 = 20000;

TCCR1A |= (1<<COM1A1)|(1<<WGM11);		//

TCCR1B |= (1<<WGM12)|(1<<WGM13)|(1<<CS11);

while (1)

{

while (PIND == 0x02)	//When pot is high, PD1 is high, thus motor turns clockwise

{

_delay_ms(20);

OCR1A = 2900;

}

//When pot is low, PD1 is low, thus motor turns counterclockwise

_delay_ms(20);

OCR1A = 400;

}

}

| 6. | SCHEMATICS |  |  |
| --- | --- | --- | --- |

TASK 1

TASK 2

TASK 3

| 7. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| --- | --- | --- | --- |

Simulations do not anything useful.

| 8. | SCREENSHOT |  |  |
| --- | --- | --- | --- |

TASK 1

TASK2

TASK3

| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| --- | --- | --- | --- |
| 1: https://www.youtube.com/watch?v=87XC5gDS9u0 2: https://www.youtube.com/watch?v=Q6BKxLB7vZs 3: https://www.youtube.com/watch?v=2VpFoAMu9oU |  |  |  |
| 10. | GOOGLECODE LINK OF THE DA |  |  |
| http:// @svn or github repository link |  |  |  |

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Emmanuel Rodriguez Lopez