Emmanuel Rodriguez Lopez

CPE301 – SPRING 2016

# Design Assignment 5

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| 1. | INITIAL CODE OF TASK 1/A |  |  |
| 2. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 2/B |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 3/C |  |  |
| 4. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 4/D |  |  |
| 5. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 5/E |  |  |
| 6. | SCHEMATICS |  |  |
| 7. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| 8. | SCREENSHOT OF EACH DEMO |  |  |
| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| 10. | GOOGLECODE LINK OF THE DA |  |  |

| 0. | COMPONENTS LIST AND CONNECTION DIAGRAM w/ PINS |  |  |
| --- | --- | --- | --- |

TASK 1 DC MOTOR

TASK 2 STEPPER MOTOR

TASK 3 SERVO

| 1 | INITIAL CODE OF TASK 1 |  |  |
| --- | --- | --- | --- |

#define F_CPU 8000000UL

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRC = 0x20;

int ADC1 = 0;

ADC_init();		//initializing ADC function

while(1)

{

ADC1 = adc_read();		//get value from adc

//From here, at each of the specified rages, if the potentiometer is turned to the area //producing these values, the motor will turn at the specified speed.

if ((ADC1 >= 901) && (ADC1 < 1024))

{

PORTC = 0x20;	//pc5

_delay_ms(10);

PORTC = 0x00;

_delay_ms(500);

}

if ((ADC1 >= 700) && (ADC1 < 901))

{

PORTC = 0x20;	//pc5

_delay_ms(75);

PORTC = 0x00;

_delay_ms(500);

}

if ((ADC1 < 700)&&(ADC1>=500))

{

PORTC = 0x20;	//pc5

_delay_ms(150);

PORTC = 0x00;

_delay_ms(500);

}

if ((ADC1 < 500)&&(ADC1>=300))

{

PORTC = 0x20;	//pc5

_delay_ms(300);

PORTC = 0x00;

_delay_ms(500);

}

if ((ADC1 < 300)&&(ADC1>=150))

{

PORTC = 0x20;	//pc5

_delay_ms(500);

PORTC = 0x00;

_delay_ms(600);

}

if ((ADC1 < 150)&&(ADC1>=0))

{

PORTC = 0x20;	//pc5

_delay_ms(600);

PORTC = 0x00;

_delay_ms(700);

}

}

return 0;

}

int adc_read()

{

ADMUX &= 0x00;

ADCSRA |= (1<<ADSC);

while((ADCSRA & (1<<ADSC)));

return ADC;

}

void ADC_init(void)

{

ADCSRA = (1<<ADEN);//|(1<<ADSC);//|(1<<ADIF);

}

| 2 | INITIAL CODE OF TASK 2 |  |  |
| --- | --- | --- | --- |

#define F_CPU 8000000UL

#include <avr/io.h>

#include <util/delay.h>

int main()

{

DDRD = 0x0F;

int ADC1 = 0;

ADC_init();			//initializing ADC function

while(1)

{

ADC1 = adc_read();	//get value from adc

//From here, at each of the specified rages, if the potentiometer is turned to the area //producing these values, the motor will turn at the specified speed.

if ((ADC1 >= 901) && (ADC1 < 1024))

{

PORTD = 0x6;		//fastest

_delay_ms(10);

PORTD = 0xC;

_delay_ms(10);

PORTD = 0x9;

_delay_ms(10);

PORTD = 0x3;

_delay_ms(10);

}

if ((ADC1 >= 700) && (ADC1 < 901))

{

PORTD = 0x6;

_delay_ms(15);

PORTD = 0xC;

_delay_ms(15);

PORTD = 0x9;

_delay_ms(15);

PORTD = 0x3;

_delay_ms(15);

}

if ((ADC1 < 700)&&(ADC1>=500))

{

PORTD = 0x6;

_delay_ms(20);

PORTD = 0xC;

_delay_ms(20);

PORTD = 0x9;

_delay_ms(20);

PORTD = 0x3;

_delay_ms(20);

}

if ((ADC1 < 500)&&(ADC1>=300))

{

PORTD = 0x6;

_delay_ms(25);

PORTD = 0xC;

_delay_ms(25);

PORTD = 0x9;

_delay_ms(25);

PORTD = 0x3;

_delay_ms(25);

}

if ((ADC1 < 300)&&(ADC1>=150))

{

PORTD = 0x6;		//slowest

_delay_ms(30);

PORTD = 0xC;

_delay_ms(30);

PORTD = 0x9;

_delay_ms(30);

PORTD = 0x3;

_delay_ms(30);

}

if ((ADC1 < 150)&&(ADC1>=0))

{

PORTD = 0x6;

_delay_ms(40);

PORTD = 0xC;

_delay_ms(40);

PORTD = 0x9;

_delay_ms(40);

PORTD = 0x3;

_delay_ms(40);

}

}

return 0;

}

int adc_read()

{

ADMUX &= 0x00;

ADCSRA |= (1<<ADSC);

while((ADCSRA & (1<<ADSC)));

return ADC;

}

void ADC_init(void)

{

ADCSRA = (1<<ADEN);//|(1<<ADSC);//|(1<<ADIF);

}

| 3 | INITIAL CODE OF TASK 3 |  |  |
| --- | --- | --- | --- |

#define F_CPU 8000000UL

#include <avr/io.h>

#include <util/delay.h>

int main()

{

DDRB = 0xFF;

ICR1 = 20000;

//Setting up PWM

TCCR1A |= (1<<COM1A1)|(1<<WGM11);

TCCR1B |= (1<<WGM12)|(1<<WGM13)|(1<<CS11);

int ADC1 = 0;

ADC_init();	//initializing ADC function

while(1)

{

ADC1 = adc_read();

// the actual value of ADC was used as the position; it is doubled and added to the //lower, 0 degree, value of the motor.

_delay_ms(20);

OCR1A = 600 + (2*ADC1);

}

return 0;

}

int adc_read()

{

ADMUX &= 0x00;

ADCSRA |= (1<<ADSC);

while((ADCSRA & (1<<ADSC)));

return ADC;

}

void ADC_init(void)

{

ADCSRA = (1<<ADEN);

}

| 6. | SCHEMATICS |  |  |
| --- | --- | --- | --- |

TASK 1

TASK 2

TASK 3

| 7. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| --- | --- | --- | --- |

Simulations do not anything useful.

| 8. | SCREENSHOT |  |  |
| --- | --- | --- | --- |

TASK 1

TASK2

TASK3

| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| --- | --- | --- | --- |
| 1: https://www.youtube.com/watch?v=3u6KKWqgKnc 2: https://www.youtube.com/watch?v=KRxYHFTMmOI 3: https://www.youtube.com/watch?v=YkqNfeDtbZg |  |  |  |
| 10. | GOOGLECODE LINK OF THE DA |  |  |
| http:// @svn or github repository link |  |  |  |

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Emmanuel Rodriguez Lopez